ROUTER01

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

2.2.2.2           1   FULL/BDR        00:00:38    172.16.1.2
GigabitEthernet0/1

Router#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

   N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

   E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

   i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

   \* - candidate default, U - per-user static route, o - ODR

   P - periodic downloaded static route

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 7 subnets, 3 masks

C       172.16.1.0/30 is directly connected, GigabitEthernet0/1

L       172.16.1.1/32 is directly connected, GigabitEthernet0/1

O IA    172.16.2.0/30 [110/2] via 172.16.1.2, 00:41:46, GigabitEthernet0/1

O IA    172.16.3.0/30 [110/3] via 172.16.1.2, 00:13:05, GigabitEthernet0/1

C       172.16.10.0/24 is directly connected, GigabitEthernet0/0.10

L       172.16.10.1/32 is directly connected, GigabitEthernet0/0.10

O IA    172.16.20.0/24 [110/4] via 172.16.1.2, 00:11:30, GigabitEthernet0/1

Router#show ip ospf interface

GigabitEthernet0/0.10 is up, line protocol is up

Internet address is 172.16.10.1/24, Area 1

Process ID 1, Router ID 1.1.1.1, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State WAITING, Priority 1

No designated router on this network

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

No Hellos (Passive interface)

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

GigabitEthernet0/1 is up, line protocol is up

Internet address is 172.16.1.1/30, Area 1

Process ID 1, Router ID 1.1.1.1, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 1.1.1.1, Interface address 172.16.1.1

Backup Designated Router (ID) 2.2.2.2, Interface address 172.16.1.2

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:08

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 2.2.2.2  (Backup Designated Router)

Suppress hello for 0 neighbor(s)

ROUTER02

Router>enable

Router#show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

1.1.1.1           1   FULL/DR         00:00:33    172.16.1.1
GigabitEthernet0/0

3.3.3.3           1   FULL/BDR        00:00:35    172.16.2.2
GigabitEthernet0/1

Router#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

   N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

   E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

   i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

   \* - candidate default, U - per-user static route, o - ODR

   P - periodic downloaded static route

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 7 subnets, 3 masks

C       172.16.1.0/30 is directly connected, GigabitEthernet0/0

L       172.16.1.2/32 is directly connected, GigabitEthernet0/0

C       172.16.2.0/30 is directly connected, GigabitEthernet0/1

L       172.16.2.1/32 is directly connected, GigabitEthernet0/1

O IA    172.16.3.0/30 [110/2] via 172.16.2.2, 00:17:54, GigabitEthernet0/1

O       172.16.10.0/24 [110/2] via 172.16.1.1, 00:53:33, GigabitEthernet0/0

O IA    172.16.20.0/24 [110/3] via 172.16.2.2, 00:16:19, GigabitEthernet0/1

Router#show ip ospf interface

GigabitEthernet0/0 is up, line protocol is up

Internet address is 172.16.1.2/30, Area 1

Process ID 1, Router ID 2.2.2.2, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State BDR, Priority 1

Designated Router (ID) 1.1.1.1, Interface address 172.16.1.1

Backup Designated Router (ID) 2.2.2.2, Interface address 172.16.1.2

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:04

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 1.1.1.1  (Designated Router)

Suppress hello for 0 neighbor(s)

GigabitEthernet0/1 is up, line protocol is up

Internet address is 172.16.2.1/30, Area 0

Process ID 1, Router ID 2.2.2.2, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 2.2.2.2, Interface address 172.16.2.1

Backup Designated Router (ID) 3.3.3.3, Interface address 172.16.2.2

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:01

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 3.3.3.3  (Backup Designated Router)

Suppress hello for 0 neighbor(s)

ROUTER03

Router>enable

Router#show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

2.2.2.2           1   FULL/DR         00:00:34    172.16.2.1
GigabitEthernet0/0

4.4.4.4           1   FULL/BDR        00:00:36    172.16.3.2
GigabitEthernet0/1

Router#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

   N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

   E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

   i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

   \* - candidate default, U - per-user static route, o - ODR

   P - periodic downloaded static route

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 7 subnets, 3 masks

O IA    172.16.1.0/30 [110/2] via 172.16.2.1, 00:45:47, GigabitEthernet0/0

C       172.16.2.0/30 is directly connected, GigabitEthernet0/0

L       172.16.2.2/32 is directly connected, GigabitEthernet0/0

C       172.16.3.0/30 is directly connected, GigabitEthernet0/1

L       172.16.3.1/32 is directly connected, GigabitEthernet0/1

O IA    172.16.10.0/24 [110/3] via 172.16.2.1, 00:45:47, GigabitEthernet0/0

O       172.16.20.0/24 [110/2] via 172.16.3.2, 00:17:45, GigabitEthernet0/1

Router#show ip ospf interface

GigabitEthernet0/0 is up, line protocol is up

Internet address is 172.16.2.2/30, Area 0

Process ID 1, Router ID 3.3.3.3, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State BDR, Priority 1

Designated Router (ID) 2.2.2.2, Interface address 172.16.2.1

Backup Designated Router (ID) 3.3.3.3, Interface address 172.16.2.2

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:00

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 2.2.2.2  (Designated Router)

Suppress hello for 0 neighbor(s)

GigabitEthernet0/1 is up, line protocol is up

Internet address is 172.16.3.1/30, Area 2

Process ID 1, Router ID 3.3.3.3, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State DR, Priority 1

Designated Router (ID) 3.3.3.3, Interface address 172.16.3.1

Backup Designated Router (ID) 4.4.4.4, Interface address 172.16.3.2

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:06

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 4.4.4.4  (Backup Designated Router)

Suppress hello for 0 neighbor(s)

Router#

ROUTER04

Router#show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

3.3.3.3           1   FULL/DR         00:00:31    172.16.3.1
GigabitEthernet0/1

Router#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

   N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

   E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

   i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

   \* - candidate default, U - per-user static route, o - ODR

   P - periodic downloaded static route

Gateway of last resort is not set

172.16.0.0/16 is variably subnetted, 7 subnets, 3 masks

O IA    172.16.1.0/30 [110/3] via 172.16.3.1, 00:18:53, GigabitEthernet0/1

O IA    172.16.2.0/30 [110/2] via 172.16.3.1, 00:18:53, GigabitEthernet0/1

C       172.16.3.0/30 is directly connected, GigabitEthernet0/1

L       172.16.3.2/32 is directly connected, GigabitEthernet0/1

O IA    172.16.10.0/24 [110/4] via 172.16.3.1, 00:18:53, GigabitEthernet0/1

C       172.16.20.0/24 is directly connected, GigabitEthernet0/0.20

L       172.16.20.1/32 is directly connected, GigabitEthernet0/0.20

Router#show ip ospf interface

GigabitEthernet0/0.20 is up, line protocol is up

Internet address is 172.16.20.1/24, Area 2

Process ID 1, Router ID 4.4.4.4, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State WAITING, Priority 1

No designated router on this network

No backup designated router on this network

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

No Hellos (Passive interface)

Index 1/1, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 0, Adjacent neighbor count is 0

Suppress hello for 0 neighbor(s)

GigabitEthernet0/1 is up, line protocol is up

Internet address is 172.16.3.2/30, Area 2

Process ID 1, Router ID 4.4.4.4, Network Type BROADCAST, Cost: 1

Transmit Delay is 1 sec, State BDR, Priority 1

Designated Router (ID) 3.3.3.3, Interface address 172.16.3.1

Backup Designated Router (ID) 4.4.4.4, Interface address 172.16.3.2

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:05

Index 2/2, flood queue length 0

Next 0x0(0)/0x0(0)

Last flood scan length is 1, maximum is 1

Last flood scan time is 0 msec, maximum is 0 msec

Neighbor Count is 1, Adjacent neighbor count is 1

Adjacent with neighbor 3.3.3.3  (Designated Router)

Suppress hello for 0 neighbor(s)

Router#